K32     PANEL CUBICLES

120      PANEL CUBICLES – UNFRAMED

Manufacturer: Formwise Washrooms Ltd, Unit 3 Wendel Point, Ryle Drive, Park Farm

Industrial Estate, Wellingborough, Northants,

NN8 6QJ. T: 01933 405961 F: 01933 405971

Email: sales@formwise-washrooms.co.uk

Web: www.formwise-washrooms.co.uk

Product reference: ‘ethos hpl’ toilet cubicles

Pilasters: Proprietary slimline pilaster profiles formed using Optisheen stainless steel. Vertical edges are fully radiused for safety with complementary stainless steel headrail fixed above pilaster profiles.

Finish: Satin Polished.

Colour: Self coloured.

Doors: 18mm moisture resistant chipboard faced both sides with high pressure laminate (HPL).

Core material: Moisture resistant chipboard (to V313).

Finish: Velour.

Colour: Colour to be confirmed from Formwise Washrooms HPL selection.

Edge treatment: Edges are lipped square.

Panels: 18mm moisture resistant chipboard faced both sides with high pressure laminate (HPL).

Core material: Moisture resistant chipboard (to V313).

Finish: Velour.

Colour: Colour to be confirmed from Formwise Washrooms HPL selection.

Edge treatment: Edges are lipped square.

Legs: Cylindrical stainless steel pedestal leg located on the partition 200mm back from face of adjoining pilaster. Adjustable to accommodate varying floor levels.

Ironmongery/ Accessories: Sprung loaded pivot hinges & dampener / Buffered coat hook / DDA compliant lever operated indicator bolt with circular face plate and emergency release facility.

Finish/ Colour: Stainless steel or stainless steel effect.

Dimensions: 2010mm overall height including 160mm floor clearance.

Other requirements: Complete with outward opening doors to ambulant and enlarged cubicles where requested. Cubicles with inward opening doors can be set out to maintain 450mm diameter manoeuvring space within the cubicle.