K32     PANEL CUBICLES

120      PANEL CUBICLES – UNFRAMED

Manufacturer: Formwise Washrooms Ltd, Unit 3 Wendel Point, Park Farm, Wellingborough, Northamptonshire NN8 6BA

T: 01933 405961 F: 01933 405971

Email: sales@formwise-washrooms.co.uk

Web: www.formwise-washrooms.co.uk

Product reference: ‘ethos sgl’ toilet cubicles

Pilasters: Proprietary slimline pilaster profiles formed using optisheen stainless steel. Vertical edges are fully radiused for safety with complementary stainless steel headrail fixed above pilaster profiles.

Finish: Satin Polished.

Colour: Self coloured.

Doors: 12/13mm solid grade laminate (SGL) to provide maximum protection against water and vandalism.

Core material: Not applicable.

Finish: Velour.

Colour: Colour to be confirmed from Formwise Washrooms SGL selection.

Edge treatment: Edges are machined to a smooth profiled finish.

Panels: 12/13mm solid grade laminate (SGL) to provide maximum protection against water and vandalism.

Core material: Not applicable.

Finish: Velour.

Colour: Colour to be confirmed from Formwise Washrooms SGL selection.

Edge treatment: Edges are machined to a smooth profiled finish.

Legs: Cylindrical stainless steel pedestal leg located on the partition 200mm back from face of adjoining pilaster. Adjustable to accommodate varying floor levels.

Ironmongery/ Accessories: Sprung loaded pivot hinges & dampener / Buffered coat hook / DDA compliant lever operated indicator bolt with circular face plate and emergency release facility. / Channel bracket partition fixings.

Finish/ Colour: Stainless steel or stainless steel effect.

Dimensions: 2010mm overall height including 160mm floor clearance.

Other requirements: Complete with outward opening doors to ambulant and enlarged cubicles where requested. Cubicles with inward opening doors can be set out to maintain 450mm diameter manoeuvring space within the cubicle.