N10 GENERAL FIXTURES/ FURNISHINGS/ EQUIPMENT

145A VANITY UNITS

Manufacturer: Formwise Washrooms Ltd, Unit 3 Wendel Point, Park Farm, Wellingborough, Northamptonshire NN8 6BA

T: 01933 405961 F: 01933 405971

Email: sales@formwise-washrooms.co.uk

Web: www.formise-washrooms.co.uk

Product reference: ‘HPL Vanity V1’ vanity unit system

Dimensions: 800mm high x max 3600mm un-jointed lengths subject to site access.

Vanity top material: 600mm bed manufactured from high pressure laminate bonded to 18mm Moisture resistant chipboard with backing laminate.

Finish / Colour: Colour from Formwise Washrooms HPL range of laminates, finish velour.

Front edge profile: Front edge postformed with 200mm downstand.

Rear edge profile: Postformed to form coved upstand up to 100mm high.

Underpanel material: 18mm moisture resistant V313 chipboard faced both sides with high pressure laminate.

Finish/colour: Colour to be confirmed from Formwise Washrooms HPL selection.

Edge treatment: All edges are lipped with matching/contrasting durable 1mm PVC.

Accessories: Concealed Keku ‘lift off’ brackets.

Support system: Treated softwood framework, notched and screwed made to measure and site assembled.

Other requirements: Not applicable.