K32     PANEL CUBICLES

120      PANEL CUBICLES - UNFRAMED

Manufacturer: Formwise Washrooms Ltd, Unit 3 Wendel Point, Ryle drive, Park Farm, Wellingborough, Northamptonshire NN8 6BA.

T: 01933 405961 F: 01933 405971

Email: sales@formwise-washrooms.co.uk

Web: www.formwise-washrooms.co.uk

Product reference: ‘lofty sgl’ toilet cubicles

Panels/ Doors & Pilasters: 12/13mm solid grade laminate (SGL) to provide maximum protection against water and vandalism.

Core material: Not applicable.

Finish: Velour.

Colour: Colour to be confirmed from Formwise Washrooms SGL selection.

Edge treatment: Edges are machined to a smooth profiled finish.

Headrail: Rigid polished aluminium section with bull-nosed leading edge.

Legs: Tubular polished aluminium.

Ironmongery/ Accessories: Polished aluminium adjustable pivot hinges / Coat hook / DDA compliant slide operated indicator bolt with elliptical face plate / Emergency release facility. Channel bracket partition fixings.

Finish/ Colour: Polished.

Dimensions: 2400mm overall height

Other requirements: Complete with outward opening doors to ambulant and enlarged cubicles where requested. Cubicles with inward opening doors can be set out to maintain 450mm diameter manoeuvring space within the cubicle.