N10 GENERAL FIXTURES/ FURNISHINGS/ EQUIPMENT

145A VANITY UNITS

Manufacturer: Formwise Washrooms Ltd, Unit 3 Wendel Point, Park Farm, Wellingborough, Northamptonshire NN8 6BA

T: 01933 405961 F: 01933 405971

Email: sales@formwise-washrooms.co.uk

Web: www.formwise-washrooms.co.uk

Product reference: ‘SGL’ vanity unit system

Dimensions: 800mm high x max 3600mm un-jointed lengths subject to site access.

Vanity tops material: 600mm wide bed manufactured from solid grade laminate (SGL).

Finish/ colour: Colour to be confirmed from Formwise Washrooms SGL selection, finish velour.

Front edge profile: Aluminium section profiled finish with a 200mm downstand.

Rear edge profile: No upstand

Underpanel material: 12/13mm solid grade laminate (SGL) to provide maximum protection against water and vandalism.

Finish/colour: Colour to be confirmed from Formwise Washrooms SGL selection, finish velour.

Edge treatment: Edges are machined to a smooth profiled finish.

Accessories: Concealed Keku ‘lift off’ brackets.

Support system: Treated softwood framework, notched and screwed made to measure and site assembled.

Other requirements: Not applicable